Thermistor Temperature Sensors & Electric Motors

An Overview of Principles, Construction, and Applications

# 1. Thermistor Temperature Sensor

## 1.1 Working Principle

A thermistor (thermally sensitive resistor) is a type of resistor whose electrical resistance changes significantly and predictably with temperature. It is fabricated from semiconductor metal oxide materials — typically mixtures of manganese, nickel, cobalt, or copper oxides — sintered into beads, discs, or rod shapes and encapsulated for protection.

The resistance-temperature relationship is highly non-linear and is described by the Steinhart-Hart equation or, for simpler approximations, the Beta (β) parameter equation. The steep resistance change per degree makes thermistors one of the most sensitive temperature sensing elements available.

## 1.2 Types of Thermistors

- NTC (Negative Temperature Coefficient) – resistance decreases as temperature rises; the most common type, used for general temperature sensing and compensation

- PTC (Positive Temperature Coefficient) – resistance increases with temperature; used for overcurrent protection, self-regulating heaters, and motor protection

- Bead thermistors – small, fast-responding; suitable for liquid immersion and medical probes

- Disc and chip thermistors – surface-mount compatible; used in PCB thermal management

- Rod and probe thermistors – rugged construction for industrial and HVAC applications

## 1.3 Key Characteristics

- High sensitivity – resistance change of 3–5% per °C for NTC types; far greater than RTDs or thermocouples

- Narrow operating range – typically −50°C to +150°C; not suited for extreme temperatures

- Non-linear output – requires linearisation in signal conditioning circuits or microcontroller firmware

- Low cost and compact – among the least expensive temperature sensors available

- Self-heating error – measurement current must be kept low to avoid heating the element and causing reading errors

- Fast thermal response – small bead types respond in under one second

## 1.4 Applications

Thermistors are widely deployed wherever accurate, low-cost temperature measurement is needed in a moderate range:

- Consumer electronics – battery temperature monitoring in laptops, smartphones, and power banks

- Medical – digital thermometers, incubator control, IV fluid temperature monitoring

- HVAC – room and duct temperature sensing, thermostat control, refrigeration monitoring

- Automotive – coolant temperature, cabin climate control, battery thermal management in EVs

- Industrial – process temperature control, transformer winding protection, 3D printer hotend sensing

- Food industry – cold chain monitoring, oven and refrigerator temperature logging

# 2. Electric Motor Actuator

## 2.1 Working Principle

An electric motor converts electrical energy into mechanical rotational energy through electromagnetic interaction. When current flows through conductors (windings) placed within a magnetic field, a force is exerted on those conductors according to the Lorentz force law (F = BIL). In a motor, this force is arranged to produce continuous torque on a rotating shaft.

The magnetic field may be produced by permanent magnets or electromagnets (field windings), and the rotor windings may be supplied via brushes and a commutator (brushed DC) or electronically commutated (brushless DC, AC induction, synchronous). The speed and torque are controlled by varying the supply voltage, current, or switching frequency.

## 2.2 Types of Electric Motors

- Brushed DC motor – simple construction, easy speed control via voltage; used in toys, power tools, and automotive accessories

- Brushless DC (BLDC) motor – electronically commutated, high efficiency, long life; used in drones, EVs, and appliances

- AC induction motor – robust, low maintenance, runs directly on AC mains; the workhorse of industrial machinery

- Permanent magnet synchronous motor (PMSM) – high efficiency and power density; used in EV traction and servo drives

- Universal motor – operates on AC or DC; high speed, compact; used in vacuum cleaners and power tools

- Servo motor – closed-loop controlled motor (typically BLDC or PMSM) with encoder feedback for precision positioning

## 2.3 Key Characteristics

- High power output – scalable from milliwatts (micro-motors) to megawatts (industrial drives)

- Continuous rotation – provides sustained rotational motion, unlike stepper or SMA actuators

- Controllable speed and torque – via PWM, variable frequency drives (VFD), or motor controllers

- High efficiency – BLDC and PMSM types exceed 90% efficiency under optimal load

- Requires feedback for precision – open-loop operation results in speed variation under load changes

- Heat generation – copper losses (I²R) and iron losses require thermal management at high loads

## 2.4 Applications

Electric motors are the most widespread actuator in the world, found in virtually every industry:

- Transportation – EV traction motors, e-bikes, electric trains, ship propulsion

- Industrial automation – conveyor belts, pumps, compressors, CNC spindle drives

- Robotics – joint drives in articulated arms, mobile robot wheel drives, gripper actuators

- Home appliances – washing machines, refrigerator compressors, fans, vacuum cleaners

- Aerospace – actuator drives for control surfaces, landing gear, and electric taxiing systems

- Renewable energy – wind turbine generators (operating in reverse as motors during grid faults)

# 3. Comparison at a Glance

The thermistor and electric motor serve fundamentally different but often complementary roles. The thermistor is a passive sensing element — it responds to thermal conditions and produces a resistance change that is converted to a signal for monitoring or closed-loop control. The electric motor is the primary output actuator — it converts electrical power into continuous mechanical rotation to drive loads. In many systems (e.g. EV battery packs, industrial drives, HVAC units) both are present: the thermistor monitors operating temperature while the motor performs the mechanical work.

| Aspect | Thermistor | Electric Motor |
| --- | --- | --- |
| Function | Sensing (thermal → elec) | Actuation (elec → rotation) |
| Energy Conversion | Temperature → resistance change | Electrical power → torque |
| Power Required | Micro-watt (bias current only) | Watts to megawatts |
| Output | Resistance / voltage signal | Continuous rotational motion |
| Control Method | Passive; signal conditioning | Voltage, PWM, or VFD control |
| Typical Use | Thermal monitoring & protection | Drives, transport, automation |

Educational Reference Document  •  Thermistor Temperature Sensors & Electric Motors